12 PHYSICS

INDUCED EMF &

POWER TRANSMISSION

INDUCED EMF

Michael Faraday discovered that when a conductor is moved in a magnetic field, an electromotive force (EMF) is set up within it.

Each of the free-moving electrons inside the metal has its own small magnetic field.  This interacts with the external magnetic field (which is changing due to the movement of the conductor), causing a force to be exerted on them.  Hence the electrons flow as a current in one direction.

The key point is the changing magnetic field - if the magnetic field stops changing (i.e. the conductor stops moving), so too does the current stop.

The induced current produces a magnetic field that interacts with the external field.  The resulting force opposes the motion of the conductor.

Work has to be done to keep the conductor moving, and this work is seen as electrical energy in the conductor.

i.e.	Mechanical work is transformed into electrical energy.

The direction of the induced current is given by the “right hand rule” - fingers in the direction of B; palm in the direction that opposes the motion of the conductor; thumb points in the direction of the induced conventional current I.

The production of an opposing force is summed up in Lenz’s Law.

Lenz’s Law:  An induced current will be in such a direction as to produce a magnetic force

that opposes the change causing it.

Examples

1.	Apply Lenz’s Law to each of the following situations.

2.	A magnet is dropped through the core of a solenoid as shown.  Graph (without calculation)

the induced current produced in the coil.

3.	A rectangular copper coil ABCD is attached by a string to a support so that it can swing

like a pendulum through a magnetic field as shown.  The coil is initially released as shown

in (a) and allowed to swing through the field.  Draw in the direction of the induced currents.

INDUCED EMF IN A SINGLE CONDUCTOR

For a single conductor moving through a magnetic field, the induced EMF is given by:

EMF (ε) =  Blv

where	         =  the induced EMF across its ends (V).

B  =  the magnetic field strength (T).

l  =  the length of conductor cutting flux lines (m).

v  =  the velocity of the conductor through the field (ms-1).

B, l and v are always at right angles to each other.

If any two of them are parallel to each other, then EMF = 0.

If any two are at an angle to each other (but not 90°), then a component will need to be used.

From this relationship, EMF can be increased by:

1.	increasing the velocity of the conductor through the magnetic field.

2.	increasing the magnetic field intensity.

3.	increasing the length of wire cutting the magnetic flux lines.

If the ends of the conductor are connected in a circuit, the induced EMF will produce a current flow (though it will fluctuate with the speed and magnetic field intensity).

For a single conductor cutting a magnetic field at right angles, it

sweeps an area given by:

A = l x vt

Example 1

Example 2

Example 3

Example 4

A yacht with a 6.00 m high mast is sailing at 4.00 ms-1 due west in the Southern Hemisphere where the Earth’s magnetic field intensity is 4.95 x 10-5 T at 54.0° dip.

(a)	Calculate the EMF induced across the ends of the mast.

(b)	Determine which end of the mast is the positive end.

Example 5

Law of Induction

Faraday demonstrated that an induce EMF is produced when magnetic flux lines are cut per unit time.

Faraday’s Law of Induction:  The size of the induced EMF depends on the rate of change of

magnetic flux linked with the coil.

EMF (ε) = -N∆Φ∆t

where	        =  the induced EMF (V).

N  =  the number of loops in the coil.

  =  the change in flux (Wb).

t  =  the time in which the change takes place (s).

Note

The negative sign shows the relative direction of the induced EMF and is related to the laws of energy conservation.

/t is called the flux linkage or rate of change of magnetic flux.  It is a measure of how many flux lines are cut by the conductor per unit time.

An induced EMF is produced by:

•	a wire or coil moving through a magnetic field.

(The more loops in the coil, the greater the induced EMF.)

•	moving a magnet in relation to a stationary wire or coil.

•	varying the magnetic field strength to alter /t.

No induced current is produced when:

•	the conductor is stationary.

•	the conductor moves parallel to the magnetic field.

Examples

1.	(Electrical Generator)

A coil of dimensions 20.0 cm x 10.0 cm and containing 200 loops is pivoted along its long

axis in a horizontal magnetic field of 0.342 T.  It is rotated at 30.0 r.p.m.  Calculate the

maximum EMF generated by the coil.

Note

This equation gives the average EMF generated in the coil, since the edge of the coil moves from cutting the maximum number of flux lines (max EMF) to cutting no flux lines (zero EMF).

Following is the derivation of the equation to give the maximum or instantaneous EMF in a rotating coil, taking into account its position relative to the magnetic field.

Consider the rotation speed of the coil.  It is represented by the angular speed , which is measured in radians s-1.

i.e.	If the angle is the angle of the coil to the vertical:

 = t

The amount of flux threading through a coil is given by:

= BA cos

= BA cos t

Using Calculus, the EMF induced due to the rate of change of the flux through the coil is given by:

One complete revolution can be thought of as:

 = 360° = 2 radians

The time taken for one revolution is the period T, and the frequency of the rotation is given by:

.



	EMF () = NBA2fsin

where	 = the angle of the coil to the vertical.

2.	Microphone and Guitar String

The function of a microphone is to convert sound

vibrations into electrical signals.

The so-called ‘dynamic’ microphone uses a tiny coil

attached to a diaphragm, which vibrates with the sound.

The coil vibrates within the magnetic field of a permanent

magnet, thus producing an induced EMF that varies with

the original sound.

The electric guitar uses electromagnetic induction

to generate an electric signal which is then amplified

electronically.

The pick-up consists of a coil around a small permanent

magnet.  The permanent magnet induces the section of the

wire above it to become a magnet.

As the string vibrates, this moving magnet induces an

EMF in the coil.  This is then amplified, and often

electronically ‘doctored’, to produce the sound required.

3.	Stationary coil - changing field.

4.	Coil pulled out of a field.

5.	Coil in a field.

6.

7.	Induced current by changing area.

Applications

Maglev Trains

Induction Cooker

THE GENERATOR

Alternating Current (A.C.)

In generators, the coil is turned mechanically (by steam, wind, water or a small petrol/diesel motor).

This induces an EMF that will produce a current flow if the coil is part of a complete circuit.

In the A.C. generator (as seen before), the maximum EMF occurs when the sides of the coil are cutting the magnetic field at right angles (when the plane of the coil is parallel to the field) and it drops to zero 1/4 of a turn later when the sides of the coil are moving parallel to the magnetic field.

Every half a turn, the EMF will return to its maximum value but the induced current direction will be reversed.

Due to the current reversing direction every half a turn, two slip rings or commutators are used to remove the induced current from the coil.

In power stations, the generators operate at 3000 r.p.m.  which equates to a frequency of 50 Hz.  Hence our power supply has its current direction changing 50 times each second.

Direct Current (D.C.)

Direct current generators work on the same principle except that a split-ring commutator is used to maintain the direction of flow of current from the coil in one direction.

Alternating Voltage and Current

An AC generator produces a current that varies sinusoidally over time with the change in magnetic flux.  In Australia, mains electricity oscillates at 50 Hz and reaches a peak voltage of   340 V.

It is often more useful to know the average power produced in a circuit so that the equations for DC electricity can be used with AC electricity.  The average power is obtained by using a value for the voltage and current equal to the peak values divided by 2.  This is referred to as the root mean square (rms) value.

Example

TRANSFORMERS

From Faraday’s formula:

it is possible to produce an induced EMF not only by having a conductor or coil moving in a constant magnetic field, but also to have the conductor stationary in a changing magnetic field.  This is the principle behind the operation of a transformer.

The structure of a transformer is as follows.

The easiest way to alter the current in the primary coil is to use AC current that constantly changes direction.

This induces an AC current in the secondary coil.

i.e.	The secondary coil is stationary in a changing magnetic field.

From the current sine wave produced by an A.C. generator, the current is changing at its maximum rate as it passes through zero while changing direction.

In the transformer, this corresponds to the maximum change in the magnetic field associated with the current in the primary coil.

Consequently, the EMF induced in the secondary coil is at a maximum at this time.

i.e. 	There is a 90° phase difference between the primary current and

secondary current.

The relationship between the number of turns and voltages in the primary and secondary coils is given by:

where 	Vp  =  voltage in the primary coil (V)

Vs  =  voltage in the secondary coil (V).

Np  =  number of turns per unit length in the primary coil.

Ns  =  number of turns per unit length in the secondary coil.

Note

The use of DC current in the primary coil will not induce an EMF in the secondary coil since the magnetic field will not change.

From the above equation, the following is true.

Step-up transformer:		Ns > Np  so   Vs > Vp

Step-down transformer:	Ns < Np  so   Vs < Vp

For a transformer, the power dissipated in the primary coil is essentially equal to the power delivered to the secondary coil.

i.e.	For calculations, transformers are assumed to be 100 % efficient.

From this can be calculated:

P (primary)  = P (secondary)

=>       VpIp  =  VsIs

Hence the relationships within a transformer can be summarised as follows.

Thus, when voltage is stepped up, the current is stepped down, and there is no power gain as a result of transformer action.

As with all machines there is some energy loss.

(See notes on Power Output.)

Example 1

A transformer has a primary winding of 200 turns that carries a current of 5.00 A at an input voltage of 48.0 V.  The secondary winding has 50 turns.

(a)	Calculate the voltage output from the transformer.

(b)	Assuming the transformer to be 100% efficient, what is the power output of the transformer?

Example 2

Power Output

Good quality transformers are usually 98-99 % efficient with 1-2 % of the energy being wasted as heat.

For this reason, electrical appliances that involve a transformer (e.g. TV, computer screens, CRO) are ventilated at the rear so this “lost heat” can escape from the machine and not cause any damage due to heat build-up.

The three main reasons for these heat losses are as follows

.

(a)	Copper losses - the resistance of the copper wires in the primary and secondary coils

cause heat losses according to the formula P = I2R.

(b)	Eddy current losses - as explained below.

(c)	Hysteresis - the magnetisation of the core is repeatedly reversed by the alternating 			  	  magnetic field.  The repeating core magnetisation process expends energy 			  	  and this energy appears as heat.  The heat generated can be kept to a 				  	  minimum by using a magnetic material that has a low hysteresis loss. 			  	  Hence, soft iron is often chosen for the core material because the magnetic 			  	  domains within it changes rapidly with low energy loss.

Example

A step-down transformer has 100 turns in its primary coil and 25 turns in its secondary coil.  The power of the transformer is 3.00 W and the voltage output is 6.00 V.

(a)	Calculate the current in the primary coil.

(b)	Explain which coil will have the thickest wire and why.

Eddy Currents

These are circulating currents of electricity that can be set up in conductors (particularly if they are large and plate-like) either moving in a constant magnetic field or stationary in a changing magnetic field.

These induced currents are closed loops that circulate in planes perpendicular to the magnetic flux.

Eddy currents will produce a magnetic field that will cause a force to oppose the motion inducing the EMF (in motors and generators) and/or dissipate energy as heat (in transformers or electromagnetic braking systems).

Transformers have laminated iron cores (made up of thin sheets) to minimise the eddy currents produced.  Thus, the amount of heat generated is small.

The planes of these laminations are set in planes parallel to the magnetic flux so that the eddy currents are confined to the width of the individual laminations.

Magnetic braking systems rely on the production of large eddy currents so that their magnetic fields can interact with the permanent magnetic field present to oppose the motion of the metal brake disc and slow the vehicle.

As the eddy current is large, large amounts of heat are generated in these braking systems.

POWER TRANSMISSION

Domestic electricity is Australia is usually generated a great distance from the areas of greatest demand within large population areas.

e.g. 	Muja power station near Collie supplies the majority of electricity to the Perth

metropolitan area some 200 km away.

Due to the great distances involved, there is a large amount of resistance within the transmission lines.  The movement of current through this resistance will cause a heating effect and hence a loss of energy.

The aim is to minimise this heat loss from the grid system by having as low a current as possible flowing.

From Year 11 work, the power produced by a current is given by:

The power produced in a generator at a power station can be considered to be constant.

From P  =  VI, in order to have a small current, V must be high.

Hence the output voltage from a power station is usually stepped-up to 330 kV or 500 kV, then stepped-down as it reaches the areas where it is needed.

Note

From P  = I2R, it can be seen that the power loss in the system is proportional to I2.

The smaller I is, the smaller the power loss due to heat.

From P  =  VI:

every 10-fold increase in V means a 10-fold decrease in I (since the power P remains

constant).

=>	a 100-fold decrease in the power loss P (since P  = I2R).

ENVIRONMENTAL IMPACT OF ELECTRICITY PRODUCTION AND TRANSMISSION

In Australia, most electricity is generated by burning coal (other sources are burning gas, hydroelectric schemes, wind power and solar cells).

The burning of coal is not very efficient as a lot of heat (≈ 50 %) is wasted.  This leads to thermal pollution of the air and water streams that are used to produce the steam to drive the turbine generators.

Gases from the burning of coal (CO2, SO2, oxides of nitrogen) add to the air pollution problems in the atmosphere.  Not only do they contribute to the Greenhouse Effect, they can all form acid rain by dissolving into rainwater.  This acid solution has killed large areas of forest and crops in North America and Europe, chemically erodes buildings and statues by reacting with the stone, and killed aquatic life in lakes and streams.  SO2 and the oxides of nitrogen are also irritants to the respiratory system and cause problems for the elderly and asthmatics.

The pylons used to hold transmission lines need to be large and strong by necessity.  However, they are a form of visual pollution for many people.

Due to the high current that flows (≈ 600 A), a strong electromagnetic field is produced.  There has been some concern in recent times that people living near these lines are more prone to certain types of cancers.  The research into this area is continuing.

The very high voltages used in transmission cause an atmospheric effect called corona phenomena.  The air breaks down around the conductors and conducts with a voltage gradient of 800 Vmm-1 of air.  Hence the lines are kept at least 700 mm from the pylons by insulators to avoid arcing.

This air leakage creates additional power losses and affects TV, cellular phone and radio communications, causes visual coronas and audible crackling.  Such effects are most prevalent when the air is moist.

CALCULATIONS INVOLVING POWER

The equations that are used are:

When considering calculations involving power from a power station, remember that:

1.	the power of the individual generators adds together to give the overall power output.

e.g.	four generators each of 15.0 MW will produce 60.0 MW output.

2.	the power output from the station is determined by the power used by each of the

houses, factories, buildings, etc. on the grid system.

Example 1

A power station generator, rated at 65.0 MW, produces a voltage of 11.0 kV that is stepped-up to 5.00 x 102 kV.  The grid system for that area has a combined resistance of 6.00 Ω.  Calculate:

(a)	the current induced in the secondary coil of the transformer.

(b)	the current produced in the grid system.

(c)	the power loss that occurs in the grid system.

Note

The above calculations are very simplistic and really only apply to D.C. current.

A.C. current is much more easily generated and transformed, so it is the preferred current.  This means that power losses are really due to impedance and inductance in the grid system.  Calculations based on these concepts are difficult to understand and so have been avoided!

Example 2

A power station sends electricity through power lines 1.00 x 102 km long and resistance

2.20 x 10-3 km-1 to a sub-station.  Here 297 kV is stepped-down to 66.0 kV for transmission to a large part of the metropolitan area.  The total resistance of this area (transmission wires, homes, factories, etc.) is 3.20 .

(a)	Calculate the power used by the area serviced by the sub-station.

(b)	Determine the power loss occurring along the transmission lines between the sub-station and

the power station.

(c)	Calculate the power required to be generated by the power station.

(d)	What would happen if the total power required by the metropolitan area was greater than the

maximum that could be generated by the power station?

Example 3

Example 4

Applications

High-voltage DC

Safety - RCD Device